| SCS / AIS | ROS2 Equivalent | Notes |
| --- | --- | --- |
| OutputInterface<T>::publish(data) | publisher->publish(msg) | Near-direct |
| InputInterface<T>::get(data) | Latest-sample via subscription callback + timer | Different paradigm — biggest trap |
| InterfaceDb::fetch("channelName") | create_publisher / create_subscription in constructor |  |
| task::Task subclass | rclcpp::Node subclass |  |
| initialize() | Node constructor or on_configure() (lifecycle node) |  |
| executive() at 10 Hz | create_wall_timer(100ms, callback) |  |
| cleanup() | Node destructor or on_shutdown() |  |
| SCS channel (typed C struct) | ROS2 .msg file | Each channel → one .msg |
| Channel FIFO queue (depth N) | QoS KEEP_LAST, depth=N | Direct mapping |
| .rb config params | declare_parameter() + YAML |  |
| task_1x() RTOS thread | std::thread with std::atomic flags |  |
| Same-ECU shared memory transport | FastDDS SHM or Cyclone SHM | Must configure explicitly |

Step-by-Step Migration

Phase 1 — Channel Inventory (do this before writing any code)

Go through every .rb config file for both apps. List every channel:

Channel Name                        Direction   C Struct Type                Queue depth
────────────────────────────────────────────────────────────────────────────────────────
LpsSaJobMgrReqstChannel             IN (JobMgr) LpsSaJobMgrReqstChannel_t   ?
LpsSaJobMgrTxChannel                OUT         LpsSaJobMgrTxChannel_t      ?
LpsSaWeighTxChannel                 IN (JobMgr) LpsSaWeighTxChannel_t       ?
LpsSaWeighReqstChannel              OUT         LpsSaWeighReqstChannel_t    ?
SwitchInputScs                      IN (HW)     SwitchInputScs_t            ?
OutputChannel                       OUT (HW)    OutputChannel_t             ?
AisJhm2TxChannel                    IN          AisJhm2Tx_t                 ?
ShmClock                            IN          ShmClock_t                  ?
...

Each channel becomes exactly one ROS2 topic. Two apps reading/writing the same channel share the same topic name.

Phase 2 — Create ROS2 Message Definitions

For each SCS channel struct, create a .msg file. Example:

SCS struct (existing C code):

struct LpsSaWeighTxChannel_t {
    uint8_t  DigStat;
    uint8_t  CalStat;
    uint8_t  DumpStat;
    float    BestBktWtInTonnes;
    uint8_t  ZeroNotifyStat;
    uint8_t  PayloadCalcMeth;
    float    Indicator;
    float    SimpleCalAdjustment;
};

ROS2 message (LpsSaWeighTx.msg):

uint8  dig_stat
uint8  cal_stat
uint8  dump_stat
float32 best_bkt_wt_in_tonnes
uint8  zero_notify_stat
uint8  payload_calc_meth
float32 indicator
float32 simple_cal_adjustment

Phase 3 — Node Skeleton (no logic yet)

class JobMgrNode : public rclcpp::Node {
public:
    JobMgrNode() : Node("lps_sa_job_mgr") {
        // All publishers
        tx_pub_    = create_publisher<lps_msgs::msg::LpsSaJobMgrTx>("lps/job_mgr_tx", rclcpp::QoS(10));
        resp_pub_  = create_publisher<lps_msgs::msg::LpsSaJobMgrResp>("lps/job_mgr_resp", rclcpp::QoS(10));
        weigh_cmd_pub_ = create_publisher<lps_msgs::msg::LpsSaWeighReqst>("lps/weigh_reqst", rclcpp::QoS(10));

        // All subscriptions — store latest in member vars
        weigh_tx_sub_ = create_subscription<lps_msgs::msg::LpsSaWeighTx>(
            "lps/weigh_tx", rclcpp::QoS(10),
            [this](const lps_msgs::msg::LpsSaWeighTx::SharedPtr msg) {
                latest_weigh_tx_ = *msg;  // just store — don't process here
            });

        reqst_sub_ = create_subscription<lps_msgs::msg::LpsSaJobMgrReqst>(
            "lps/job_mgr_reqst", rclcpp::QoS(10),
            [this](const lps_msgs::msg::LpsSaJobMgrReqst::SharedPtr msg) {
                latest_reqst_ = *msg;
            });

        // The executive — 100ms = 10 Hz
        timer_ = create_wall_timer(std::chrono::milliseconds(100),
            std::bind(&JobMgrNode::executive, this));
    }

private:
    void executive() {
        scs_rx();
        pt_update();
        send_cmd_to_weigh_app();
        handle_tipoff_and_horn();
        process_store();
        scs_tx();
    }
    // ...
};

This skeleton exactly mirrors the AIS task structure. The timer IS the executive.

Phase 4 — Port ScsRx and ScsTx (thin translation layer)

ScsRx() becomes: copy from latest_* ROS2 messages into LpsPtInputs_t

void scs_rx() {
    // Populate pass tracker inputs from latest received ROS2 messages
    pt_inputs_.DigStat        = latest_weigh_tx_.dig_stat;
    pt_inputs_.DumpStat       = latest_weigh_tx_.dump_stat;
    pt_inputs_.BestBktWt      = latest_weigh_tx_.best_bkt_wt_in_tonnes;
    pt_inputs_.StoreRequest   = latest_reqst_.store_reqst;
    pt_inputs_.ZeroRequest    = latest_reqst_.zero_reqst;
    // ... etc
}

ScsTx() becomes: copy from LpsPtOutputs_t into ROS2 message and publish

void scs_tx() {
    lps_msgs::msg::LpsSaJobMgrTx msg;
    msg.pass_count       = pt_outputs_.PassCount;
    msg.truck_weight     = pt_outputs_.TruckWeight;
    msg.disp_best_bkt_wt = pt_outputs_.DispBestBktWt;
    // ... etc
    tx_pub_->publish(msg);
}

The business logic call in between stays completely unchanged:

void pt_update() {
    weigh_mode(pt_inputs_);   // exact same library call, no modification
    pt_outputs_ = /* library result */;
}

Phase 5 — NVM Thread

// In node constructor
nvm_thread_ = std::thread(&JobMgrNode::nvm_task, this);

void nvm_task() {
    while (running_) {
        if (machine_config_write_flag_.exchange(false)) {
            app_nvm_jobmgr_machine_spec_config_file_write(...);
        }
        if (live_values_write_flag_.exchange(false)) {
            app_nvm_jobmgr_live_values_file_write(...);
        }
        std::this_thread::sleep_for(std::chrono::milliseconds(50));
    }
}

The flag mechanism from AIS maps directly. std::atomic<bool> replaces RTOS flags.

What Will Create Real Issues

1. Timing determinism — HIGH RISK

Problem: AIS runs on an RTOS with hard 10 Hz scheduling. Linux (which ROS2 runs on) is not real-time. Your 100ms timer can fire at 105ms or 98ms. The weighing library has ExecRate: 0.02s (20ms) — if you're feeding it data with variable timing, weight calculations drift.

What to do:

Use a real-time Linux kernel (PREEMPT_RT patch) on the target ECU

Set the executor thread to SCHED_FIFO priority:

struct sched_param param;
param.sched_priority = 80;
pthread_setschedparam(pthread_self(), SCHED_FIFO, &param);

If strict real-time isn't achievable, pass timestamps into the weighing library so it can compensate for variable deltas

2. The get() vs subscription callback paradigm — SILENT BUGS

Problem: In SCS, input->get(data) returns the latest sample from the channel. If no new data arrived since last cycle, you get the previous cycle's value — this is expected and safe (the algorithm runs every cycle regardless).

In ROS2, if you use callbacks naively, the callback only fires when new data arrives. If WeighApp publishes at 10 Hz and JobMgr timer fires at 10 Hz but they drift, you'll sometimes process two-cycle-old data or miss a cycle entirely — and you won't know.

What to do: Use the latest-sample pattern shown in Phase 3. Initialize all latest_* member variables to safe defaults (zeros or NVM-loaded values) before starting the timer. The behavior then matches SCS exactly.

3. Hardware I/O channels — NEEDS SEPARATE WORK

Problem: SwitchInputScsInput (STG4 button) and OutputChannelOutput (PORT_SINK_3 horn) are hardware interfaces, not inter-app SCS channels. In AIS they're abstracted by the BSP. In ROS2 there's no equivalent HAL.

What to do: Create a separate HalNode that:

Reads GPIO for STG4, publishes to lps/switch_input topic

Subscribes to lps/output_channel topic, drives GPIO for horn relay

JobMgr then treats these as normal ROS2 topics — no different from WeighApp channels. But the HalNode needs to talk to actual hardware (via /dev/gpiochip, libgpiod, or the target ECU's BSP).

Same issue for PwmInputChannels in WeighApp — the sensor PWM signals need a driver node.

4. Same-machine DDS overhead — MEDIUM RISK

Problem: In SCS, JobMgr and WeighApp are on the same ECU communicating through shared memory — essentially zero overhead. In ROS2 with default FastDDS, even loopback goes through the DDS stack (~0.1–1ms per message).

At 10 Hz (100ms cycle) this is fine. But with 8+ channels publishing every 100ms, DDS discovery and keep-alive traffic adds up.

What to do: Configure FastDDS shared memory transport in fastdds_profile.xml:

<transport_descriptor>
    <transport_id>shm_transport</transport_id>
    <type>SHM</type>
</transport_descriptor>

This brings latency back to microseconds for same-machine communication.

5. NvmInitialize() blocking — startup sequencing

Problem: AIS initialize() calls NvmInitialize() which blocks until NVM is read. In ROS2 you can't block in the node constructor — it prevents the executor from spinning, so subscriptions never get data, so NVM never arrives.

What to do: Use ROS2 Lifecycle Nodes:

on_configure(): start NVM read, return SUCCESS only when done

on_activate(): start the timer (executive loop)

on_deactivate() / on_cleanup(): stop timer, flush NVM

This exactly mirrors the AIS lifecycle with proper blocking semantics.

6. Request/Response channels — design decision needed

Problem: LpsSaJobMgrReqstChannel + LpsSaJobMgrRespChannel is a request/response pattern. In SCS this is two separate channels. In ROS2 you have a choice:

Option A — Two topics (simpler, matches SCS 1:1): UI publishes request to lps/job_mgr_reqst, JobMgr publishes response to lps/job_mgr_resp. No synchronization guarantee.

Option B — ROS2 Service: UI calls lps/store_payload service, waits for response. Proper synchronous request/response. But then the 10 Hz timer can't service the request mid-cycle — need careful design.

Recommendation: Start with Option A (two topics) for the initial port. It matches the existing behavior exactly. Migrate to services later if needed.

7. Machine and DataLink channels — scope clarification needed

Problem: MachineInput, DataLinkDataInput, SEAStatusInput (WeighApp inputs) are large, complex channels from other ECU applications (Machine App, CDL/DataLink App). These apps aren't part of your porting scope.

In the real system these will need their own ROS2 wrapper nodes or drivers. For initial development/testing, you'll need to mock these — publish test data from a separate node.

Recommended Migration Order

Create all .msg files for every SCS channel (both apps)

Port JobMgr first — simpler than WeighApp (no sensor processing, no calibration library)

Mock WeighApp — publish fake LpsSaWeighTx data from a test node, verify JobMgr state machine responds correctly

Port WeighApp — replace the mock, integrate lps_weighing and lps_cal

HAL nodes last — once logic is validated, add real hardware I/O

This lets you validate the business logic migration independently of the hardware challenge.